# Objective

This example demonstrates how to dynamically enable or disable a GATT Service on the go.

# Overview

- In this example the BLE Component is configured as a GAP Peripheral and a GATT Server. It supports a custom service called the RGB LED through which the Central can control the RGB Led on PSoC 4 BLE Pioneer Kit. This service can be enabled or disabled dynamically by sending a command from the UART terminal to the PSoC 4 BLE Component.

# Requirements

Design Tool: PSoC Creator 3.1 SP1, CySmart 1.0

Programming Language: C (GCC 4.8.4 – included with PSoC Creator)

Associated Devices: All PSoC 4 BLE devices

Required Hardware: CY8CKIT-042-BLE Bluetooth® Low Energy (BLE) Pioneer Kit

# PSoC Creator Schematic

# Hardware Setup

# The CY8CKIT-042-BLE Baseboard acts as a BLE Peripheral and the BLE Dongle acts as the BLE Central.

# Firmware:

BLE Component is configured as a GAP Peripheral and a GATT Server. A custom profile RGB LED is added

Initially, the BLE Component and UART Component are turned on. The BLE Component starts advertising as soon as the BLE Stack is ON. The while loop in the main function continuously scans for any inputs via UART terminal. If ‘D’ is received, the CyBle_GattsDisableAttribute API is called for disabling the RGB LED custom service. If ‘E’ is received, the CyBle_GattsEnableAttribute API is called for enabling the RGB LED custom service. A “Service Changed” notification is sent to the master, if a connection is already established. This custom service is enabled by default.

The custom service enables the control of RGB LEDs from the master. When the master writes a value to the RGB LED custom service attribute, the CYBLE_EVT_GATTS_WRITE_REQ event is received by the Peripheral. Upon the reception of this event, the data is extracted and the status of the LED is updated. Refer Table 1 to see the list of values and corresponding RGB LEDs state. Also, a response is sent to the central acknowledging the reception of the write request.

## Build and Program

This section shows how to build the project and program the PSoC 4 BLE device on the CY8CKIT-042-BLE Baseboard.

- Open PSoC creator 3.1. Go to File -> Open -> Project / Workspace. Browse for the folder containing the project files and select BLE_Dynamic_GATT_Service_Configuration.cyprj.

- Go to Build -> Build BLE_ Dynamic_GATT_Service_Configuration.

- On a successful build, the total flash and SRAM usage is reported as shown below

- Select Debug > Select Debug Target, as shown below.

Selecting Debug Target

- In the Select Debug Target dialog box, click Port Acquire, and then click Connect as shown below. Click OK to close the dialog box.

- Select Debug > Program to program the device with the project, as shown below.

You can view the programming status on the PSoC Creator status bar (lower-left corner of the window), as shown below.

# Operation and Testing

- After programming, connected the kit to your PC, open a serial terminal (putty for example) corresponding to the BLE Pioneer Kit baseboard and configure the baud rate as 115200. Press the reset switch (SW1) on the Kit. You can see the “BLE Component ON” and “Starting to advertise” messages.

- On your PC, launch CySmart 1.0. It is located in the All Programs -> Cypress -> CySmart folder in the Windows start menu. The tool opens up and asks you to Select BLE Dongle Target. Select the Cypress BLE Dongle (COMxx) and click Connect, as shown in below.

CySmart: Select BLE Dongle Target

- When the BLE Dongle is connected, click on Start Scan to search for your BLE device as shown below.

- You can see the “BLE_DGSC” device detected as shown below. Click the Connect button.

- After connection, click Discover All Attributes and “Enable All Notifications” buttons. The window will now appear as shown below. You can see that 3 services are enabled as shown in the figure below. The first two services are the default Generic Access Service and Generic Attribute Service. The third one is the RGB LED custom service included in this example. This shows that the third service is currently enabled.

- The status of the RGB LEDs can be changed by the CySmart using this custom service. Select the row corresponding to the handle 0x000E. On the right pane, in the Attribute Details Tab, enter a number from 1 to 7 and click Write Value. You will see the state of RGB LEDs change on the BLE Pioneer Kit baseboard as per the Table below:

Table 1: RGB LED Custom Characteristic Value and the State of the LEDs

| Value | RED Led | Green LED | BLUE Led | Resultant Color |
| --- | --- | --- | --- | --- |
| 0 | OFF | OFF | OFF | - |
| 1 | ON | OFF | OFF | RED |
| 2 | OFF | ON | OFF | GREEN |
| 3 | ON | ON | OFF | YELLOW |
| 4 | OFF | OFF | ON | BLUE |
| 5 | ON | OFF | ON | MAGENTA |
| 6 | OFF | ON | ON | CYAN |
| 7 | ON | ON | ON | WHITE |

Also, when read, the new value will be updated on the CySmart as shown below:

- To disable the service, Enter ‘D’ from the UART terminal. The debug messages will be displayed as shown.

- In CySmart, click the Clear button and then Click Discover All Attributes button. This time, the custom service will not be shown.

- To enable the service again, enter ‘E’ on the UART terminal. The debug messages will be displayed as shown below:

- Clicking Discover All Attributes button on CySmart again will show the service indicating that it has been re-enabled.

# Related Documents

The table below lists all relevant application notes, code examples, knowledge base articles, device datasheets, and Component / user module datasheets.

Related Documents

| Document | Title | Comment |
| --- | --- | --- |
| AN91267 | Getting Started with PSoC® 4 BLE | A guide for beginners on PSoC 4 BLE |
| 001-90479 | Programmable System-on-Chip (PSoC®) | PSoC® 4: PSoC 4XX7_BLE Family Datasheet |